# SGLang Layerwise Offloading Analysis: Wan2.2 Video Generation

Date: February 2026 Hardware: ACES cluster, 4x/6x H100 PCIe GPUs, 488GB RAM Software: SGLang (PR #15511), Wan2.2-T2V-A14B-Diffusers (dual 14B transformers)

## 1. Background

SGLang PR #15511 introduces a new layerwise offloading strategy (dit_layerwise_offload) for video diffusion models. We evaluated it against the existing full-model offloading (dit_cpu_offload) to understand the performance tradeoffs on PCIe-based GPU systems.

Wan2.2-T2V-A14B uses two 14B-parameter transformers in a mixture-of-experts (MoE) configuration at the timestep level: one processes high-noise steps, the other handles low-noise steps. During a 27-step denoising run, the model switches from one transformer to the other at a boundary determined by flow_shift and boundary_ratio.

## 2. How the Two Offload Strategies Work

We verified both implementations directly from source code with line-number citations.

### Full-Model Offload (Old)

- Transfers the entire transformer (~28GB) between CPU and GPU using blocking .to(device) calls

- At the transformer switch point, the active model moves to CPU and the other moves to GPU

- Uses PyTorch FSDP CPUOffloadPolicy for automatic weight management

- One large spike in latency at the switch point, but fast steady-state steps

### Layerwise Offload (New, PR #15511)

- Transfers one layer at a time (~700MB) using a dedicated CUDA copy stream with async H2D prefetch

- CPU pinned memory buffer is the source of truth – no GPU-to-CPU (D2H) copies ever occur

- Pre-hooks prefetch the next layer(s) before they’re needed; post-hooks release GPU memory immediately

- No spike at the switch point, but adds ~2s overhead per step on our hardware

### Comparison

| Aspect | Full-Model (Old) | Layerwise (New) |
| --- | --- | --- |
| Transfer granularity | Entire transformer (~28GB) | Per-layer (~700MB) |
| Transfer method | Blocking .to() | Async H2D on dedicated stream |
| GPU-to-CPU copy | Yes (full model) | None (CPU buffer unchanged) |
| Peak VRAM | ~61 GB | ~23 GB |
| Switch point cost | ~8s blocking spike | No spike |
| Per-step overhead | Minimal | ~2s on PCIe hardware |

## 3. PCIe Limitation: Why Layerwise Offload Underperforms on Our Hardware

PR #15511’s async prefetch strategy is designed to overlap data transfers with computation. This works on NVLink systems (like H100 SXM) where GPU-to-GPU communication (used by Ulysses sequence parallelism) travels over NVLink while CPU-to-GPU transfers use PCIe – two separate physical paths.

On ACES H100 PCIe, both GPU-GPU and CPU-GPU transfers share the same PCIe fabric (~64 GB/s unidirectional). The async prefetch cannot overlap with Ulysses all-to-all communication because they compete for the same bandwidth.

| System | GPU-GPU Path | CPU-GPU Path | Can Overlap? |
| --- | --- | --- | --- |
| H100 SXM (NVLink) | NVLink (900 GB/s) | PCIe (128 GB/s) | Yes |
| H100 PCIe (ACES) | PCIe (128 GB/s) | PCIe (128 GB/s) | No |

This is likely why PR #15511’s benchmarks (run on NVLink systems) show better performance than what we observe.

## 4. Preliminary Benchmark Results (4 GPUs)

| Metric | Layerwise (New) | Full-Model (Old) |
| --- | --- | --- |
| Total time | 12:00 | 11:12 |
| Denoising time | 788.2s | 714.1s |
| Steady-state per step | ~23s | ~21s |
| Switch step spike | None | ~8s |
| Peak GPU memory | 22.88 GB | 61.25 GB |

Old offload was ~7% faster on 4 GPUs despite having a blocking transfer spike. The new offload’s per-step overhead (~2s x 26 steps = ~52s total) exceeds the old offload’s one-time spike cost.

However, the new offload uses 2.7x less GPU memory (23 GB vs 61 GB), which is its primary design goal for memory-constrained deployments.

### Caveat: Config Issue Discovered

These benchmarks ran with an incorrect flow_shift parameter (3.0 instead of 12.0) due to a SGLang model config resolution bug when using local model paths. This caused the transformer switch to happen at step 10 instead of the correct step 19. We are re-running benchmarks with the correct config. The 7% figure may change.

See Section 5 for details.

## 5. Config Resolution Bug Found

We discovered that SGLang’s model registry does not correctly resolve local model paths for Wan2.2-A14B.

The issue: When using --model-path ./models/wan2.2, SGLang’s registry (registry.py) cannot match the path to the correct Wan2_2_T2V_A14B_Config class. It falls through to a generic Wan config (WanT2V480PConfig) which sets flow_shift=3.0 instead of the correct 12.0.

Why it matters: flow_shift controls the denoising timestep schedule. With the wrong value, the transformer switch point changes:

| flow_shift | Switch step (out of 27) | Config class |
| --- | --- | --- |
| 3.0 (incorrect) | Step 10 | WanT2V480PConfig (generic) |
| 12.0 (correct) | Step 19 | Wan2_2_T2V_A14B_Config |

Resolution: Use the HuggingFace model identifier (Wan-AI/Wan2.2-T2V-A14B-Diffusers) instead of a local directory path. This matches the exact path registered in SGLang’s config registry. All benchmark scripts have been updated.

## 6. Next Steps

- Re-run benchmarks with corrected model path (4-GPU and 6-GPU, both offload strategies)

- 6-GPU scaling comparison – does layerwise offload become competitive with more GPUs?

- Add timestep logging to confirm correct config is loaded (flow_shift=12.0)

- Profile PCIe bandwidth during offload to quantify contention between Ulysses and H2D transfers

## 7. Key Takeaways

- Layerwise offloading (PR #15511) trades speed for memory – 2.7x less VRAM but ~7% slower on 4 PCIe GPUs (pending re-validation)

- The “zero-cost” prefetch claim requires NVLink for compute/transfer overlap; PCIe systems see per-step overhead

- A config resolution bug in SGLang affected our benchmark parameters – discovered during source code verification

- Corrected benchmarks are queued for re-run